SECTION 08360

SECTIONAL OVERHEAD DOORS

PART 1 GENERAL

1.1     SECTION INCLUDES

A.     Sectional  Overhead  Doors

1.2     RELATED SECTIONS

A.     Section 04810-Unit Masonry Assemblies

B.     Section 05100-Structural Metal Framing

C .    Section 06100- Rough Carpentry

D.     Section 09900- Paints and Coatings

E.     Section 16050- Wiring Connections

1.3     REFERENCES

A.     ANSI/DASMA 102-American National Standard Specifications for Sectional Overhead

Type Doors.

1.4     SUBMITTALS

A.     Submit under provisions of Section 01300

B.     Shop Drawings: Show in detail opening and clearance dimensions. Include elevations

of sections and track. Include section finish, gauge and non standard options.

Show detail of jamb material and connections.

C.     Verification Samples: Two samples, minimum 6 inches square, representing actual

actual product selected.

D.     Operation and Maintenance data.

1.5     QUALITY ASSSURANCE

A.     Manufacturer Qualifications: Company specializing in manufacturing products in this

08360-1

section with minimum five years experience.

B.     Installer Qualifications: Authorized representative  of the manufacturer with minimum

five years documented experience.

1.6      DELIVERY, STORAGE AND HANDLING

A.     Store products in manufacturer’s unopened packaging until installation.

B.     Store materials in a dry, weathertight  location.

PART 2 PRODUCTS

2.1     MANUFACTURERS

Acceptable Manufacturers: General Doors Corporation, which is located at One Monroe St.;

Bristol, PA 19007; Tel: 215-788-9277; Fax 215-788-9450; Email:

sales@general-doors.com; Web: www.general-doors.com

2.2     FULL-VIEW ALUMINUM SECTIONAL OVERHEAD DOORS

A.     General Doors Corporation Model Full-View Aluminum.

1.     Door Assembly: Extruded 6053-T5 Aluminum. Doors

comply with:

a.     ANSI/DASMA 102-2011 American National Standard Specifications for

Sectional Overhead Type Doors.

2.     Door Model: Full-View Aluminum

3.     Frame Thickness: 1.75 inches

4.     Finish:

a.  clear anodized

b.  bronze anodized

c.  pre-painted white

d.  custom powder coat or anodized

5.     Solid Panels:

a.  smooth .050” thick aluminum

b.  embossed .017 thick aluminum laminated to ½” thick polystyrene

c.  embossed .017 thick aluminum laminated to ½” thick plywood

6.     Solid Sections:

a.  Model Weather Breaker 175 section

7.     Glazing:

a.  open for glazing by customer

b.  1/8” DSB glass

c.  1/8” tempered glass

d.  ¼” clear laminated(safety) glass

e.  ¼” white laminated(safety) glass

f.  1/8” acrylic(Plexiglass)

g.  1/8” polycarbonate(lexan)

h.  ½” insulated glass

i.  ½” tempered insulated glass

j.  ¼" twin wall(ribbed) polycarbonate

8.     Bottom Rail:

a.  3-3/8 inch

b.  5-3/8 inch

c.  7-3/8 inch

9.     Top Rail:

a. 3-3/8 inch

b. 5-3/8 inch

10.     Meeting Rails: 3-3/4 inch combined

11.    Center Stiles:

a. 2 inch

b. 3-1/2 inch

08360-2

12.    Reinforcing: Integral fin at each intermediate rail.

13.    End Stiles:

a. 3-3/8 inch

b. 5-3/8 inch

c. 7-3/8 inch

14.    Torsion Springs:

a. 10,000 cycles

b. 25,000 cycles

c. 50,000 cycles

d. 75,000 cycles

e. 100,000 cycles

1. Mounted on 1’’ tube (14 ga. Min)

2. Mounted on 1’’ CRS solid shaft keyed full length

15.    Track and Operating Hardware:

a. Standard lift

b. High lift

c. Vertical Lift

d. Low headroom

e. Roof incline

f. High lift w/ Roof Incline

16.    Hardware: Heavy duty galvanized hinges and fixtures. Floating ball bearing

rollers with hardened steel races. Galvanized aircraft cables, 7x19

construction, minimum 5:1 safety factor.

17.    Lock:

a. Inside slide, spring activated

b. Exterior keyed cylinder with lock bars

18.    Track:

a. 2 inches wide, roll formed galvanized

b. 3 inches wide, roll formed galvanized

19.    Seals:

a. Vinyl weatherseal full width of door, bottom section.

b. Standard continuous, replaceable dual seals between sections.

2.3     FABRICATION

A.     Check site dimensions prior to fabrication

PART 3 EXECUTION

3.1     EXAMINATION

A.     Verify opening dimensions, jambs plumb, level and square

3.2     INSTALLATION

A.     Install assembly in accordance with manufacturer’s instructions.

B.     Assemble work plumb, true, square, straight , level and accurate as per drawings

C.     Position jamb weather seal to contact door when closed.

3.4     ADJUSTING

A.     Adjust door to ensure smooth operation through open and close cycle. Use

manufacturer  approved lubricant on all bearings.

08360-3

3.5     DEMONSTRATION

A.     Demonstrate proper operation to user.

END OF SECTION

08360-4